# Electronics Documentation

### 1. Electronics Block Diagram

### 2. Components Table

| Component | Quantity | Purpose | Justification |
| --- | --- | --- | --- |
| ESP32 | 1 | Main microcontroller to process inputs and control outputs. | multiple GPIO pins, built-in Bluetooth capability, and ability to handle simultaneous inputs and outputs efficiently |
| Xbox Controller | 1 | To control laser aiming and user inputs (trigger + buttons) | Replaced joystick module due to centering issues; provides smoother, more reliable, and precise control along with better user ergonomics |
| Servo Motors | 4 | 2 for laser movement (X & Y), 2 for target pop-up | Servos allow controlled angular movement, making them suitable for directional aiming and repeated pop-up actions |
| Laser Module | 1 | To act as the shooting mechanism. | Provides a visible and precise beam for accurate targeting and interaction. |
| LDR Sensor (5mm) | 2 | To detect headshots. | Smaller size enables precise targeting, increasing difficulty and skill differentiation |
| LDR Sensor (12mm) | 2 | To detect body shots | Larger sensing area makes detection easier, balancing gameplay difficulty |
| NeoPixel Ring (16 LEDs) | 1 | To display sequence, feedback, and game status | Individually addressable LEDs allow dynamic sequence visualization and real-time feedback |
| LED Strips | 2 | To provide additional visual feedback. | Enhances immersion and makes feedback more noticeable |
| LCD Display (I2C) | 1 | To display instructions, prompts, and score | Ensures clear communication with the user, making the system intuitive |
| Buzzer | 1 | To provide auditory feedback. | Gives immediate sound-based feedback for hits, misses, and game states |
| IR Sensor | 1 | To detect controller pickup and reset state. | Enables automatic system activation and reset without additional inputs |
| Breadboard & Wires | Multiple | To connect all components | Allows flexible prototyping and quick modifications during development |
| PCA9685 Servo Driver | 1 | To control multiple servos using PWM | Reduces load on ESP32 and allows stable, precise control of multiple servos simultaneously |
| LM2596 Buck Converter | 1 | To regulate voltage supply to components | Ensures stable voltage output and protects components from over-voltage |
| DC Power Adapter | 1 | To supply power to the system | Provides consistent and sufficient power for all components |
| Push Buttons | 2 | Used during the recall phase for the user to replicate the sequence of targets hit | Provides a simple, reliable, and low-latency input method for sequence replication, eliminating dependence on the controller. |