# Test Plan & Results

### Test Plan

| Test | Method | Expected Result |
| --- | --- | --- |
| Stepper motor individual test with DRV8825 | Connected NEMA 17 stepper motor to DRV8825 driver and controlled STEP and DIR pins via ESP32 code to rotate in both directions | Motor should rotate smoothly in clockwise and anticlockwise directions without stalling |
| Joystick module centering test | Read analog values from joystick X and Y axes using ESP32 and observed values when joystick is untouched | Values should remain stable around a fixed center point (neutral position) |
| PCA9685 servo driver (4 servos) initial test | Connected 4 servos to PCA9685 and attempted to control via ESP32 without proper library setup | All 4 servos should respond correctly to control signals |
| PCA9685 with proper library setup | Installed required PCA9685 libraries and re-tested control of 4 servos | All servos should move accurately and respond to commands without issues |
| LDR module modification test | Cut the existing LDR module and soldered a 12mm analog LDR sensor in place of the original 5mm sensor | Modified LDR should detect light/laser input correctly with improved sensing area |
| LCD individual test | Connected LCD to ESP32 and ran test code to display text | LCD should display text clearly without glitches |
| Linear rail and belt system test | Integrated stepper motor with GT2 belt and linear rail shaft and tested movement under load | System should move smoothly along the rail and belt should handle load without slipping or stalling |
| Xbox controller input test (Python) | Ran a local Python script to read analog values from the Xbox controller joystick inputs | Analog values should be read correctly and reflect joystick movement accurately |
| ESP32 servo control through main.py | Uploaded main.py to the ESP32 to read incoming control values and move the servos accordingly | Servos should respond correctly to the received input values and move smoothly in the intended direction |
| Breadboard circuit test | Assembled full circuit on breadboard connecting ESP32, sensors, drivers, and output components | All components should power on and communicate correctly without loose connections or faults |

### 2. User Testing

- Who were the users?

We were the primary users during the initial stages of testing. Later, Affan was involved as a user to test the interaction with the controller and laser system.

- What tasks were given?

Initially, we tested the core game logic using the breadboard circuit setup. This involved verifying whether the sequence display, input, and response system worked as intended.

After that, Affan was given the task of using the controller to move the laser and aim at the LDR targets. This test focused on checking the accuracy of laser movement, responsiveness of the control system, and whether the LDRs correctly detected the laser hits.

### 3. Results

| Test | Outcome | Pass/Fail |
| --- | --- | --- |
| Stepper motor individual test with DRV8825 | Motor rotated successfully in both directions when tested individually | Pass |
| Joystick module centering test | Joystick did not return consistent center values and showed drift | Fail |
| PCA9685 servo driver (4 servos) initial test | Servos did not respond correctly due to missing library setup | Fail |
| PCA9685 with proper library setup | After installing libraries, all 4 servos worked properly and responded as expected | Pass |
| LDR module modification test | Modified module successfully detected light input after replacing with 12mm LDR | Pass |
| LCD individual test | LCD displayed text correctly during testing | Pass |
| Linear rail and belt system test | System showed heavy vibration, inconsistent movement, and belt failed to pull the load due to stepper driver and torque limitations | Fail |
| Xbox controller input test (Python) | Python script successfully read analog values from the controller and reflected movement accurately | Pass |
| ESP32 servo control through main.py | main.py controlled the servos correctly with a slight delay in response, but overall functionality was accurate | Pass |
| Breadboard circuit test | Complete circuit worked as expected with stable connections and proper communication between components | Pass |

### 4. Observations

- User Behavior

The users were able to follow the game logic smoothly, and the “ .append()” function that increased the level difficulty worked well. As the sequence grew, the interaction remained understandable and the progression of difficulty was clear.

- Pain Points

A noticeable pain point was the slight delay in the controller response, since the PC was acting as a bridge between the controller and the system. Although the delay was not severe enough to make the experience frustrating, it was still noticeable during interaction.

### 6. Improvements Suggested

Based on testing, one improvement was to use a separate ESP32 to handle the laser movement, while the PC continues to act as the bridge. This would help distribute the processing load and improve the responsiveness of the system.

Another improvement considered was to explore alternative methods of controlling the servos, such as using a joystick module. However, this was not implemented effectively since the joystick module did not perform reliably during testing.

Push buttons were also suggested as a simpler and more reliable input method. However, the ergonomics and feel of the controller made the game more dynamic, so the controller was preferred. While the controller input was successfully implemented in code, the delay caused by the PC acting as a bridge was not fully accounted for.