# Software Documentation

### 1. Software Overview

• What does the software do?
The software controls an interactive laser-based shooting game that combines real-time physical input with memory-based gameplay. The system is divided across two ESP32 microcontrollers to handle different tasks efficiently.

The first ESP32 runs the core game logic. It generates random target sequences, manages scoring, and controls feedback through the LCD display, NeoPixel LEDs, and buzzer. It also processes input from LDR sensors to detect hits and verifies the user’s response during the sequence replication phase.

The second ESP32 is dedicated to input control. It receives joystick data and translates it into movement for a dual-axis servo mechanism, allowing the user to aim the laser module smoothly in real time. This separation ensures responsive control while keeping the game logic stable.

Together, the system creates a seamless interaction where the user aims using the controller, shoots targets in a specific sequence, and then recalls that sequence using buttons.

### 2. System Logic

System Flow

User picks up controller

IR sensor detects → game starts

LCD shows READY → STEADY → GO

ESP32 generates a random sequence

LCD displays the sequence

User aims laser using joystick

User shoots targets in the given order

LDR sensors detect hits

ESP32 assigns score (Headshot +5 / Shot +3)

After sequence is completed → LCD shows REPLICATE

User inputs sequence using button1 / button2

ESP32 compares input with original sequence

If correct → LEDs turn green → next level

If wrong → LEDs turn red + buzzer → game over

Final score displayed on LCD

If all levels completed → celebration LEDs

LCD shows PLACE CONTROLLER

System resets when controller is placed back

### 3. Flowchart

Start
Controller picked up (IR sensor detects)
Display READY → STEADY → GO
Initialize score and level

Generate random sequence
Display sequence on LCD

Shooting Phase
User aims laser and shoots targets in order
LDR detects hit
Add score (Headshot +5 / Shot +3)
Repeat until sequence completed

Replication Phase
Prompt user to replicate sequence
User inputs using button1 / button2
Compare input with original sequence

Display final score
Show celebration LEDs (if won)
Display “PLACE CONTROLLER”
Wait for controller to be placed back