# BULLSEYE! (Last moment failure of project)

# Project Design Document

### 1. Design Intent

- What experience are you trying to create?

We are creating an interactive, skill-based laser shooting experience that simulates a real-life shooting range through dynamic difficulty levels and real-time feedback.

- Who is it for?

It is designed for young users seeking an engaging, hands-on alternative to screen based gaming, combining physical interaction with competitive and immersive play.

### 2. Inspiration & References

- Moving targets: https://youtube.com/shorts/h6fAe5TdDjY?si=gHuJRhVWp05Lvmf9

From our inspiration, we decided to make a shooting range with targets moving in two axes- x axis and z axis, but with a twist. You don’t see the targets moving. You only get to see the target once it pops up in random locations.

### 3. User Journey

Step-by-step experience of the user:

- User Starts — Activation

The system begins in an idle state and guides the user into the experience.

- LCD displays “INITIALISING…”

- Stepper motors move to the home position (0,0) using limit switches

Once initialized:

- LCD displays “PICK UP JOYSTICK”

When the user picks up the controller:

- IR sensor detects absence

- LCD updates to “GET READY”

A countdown begins:

- LCD displays “3… 2… 1…”

- Neopixel flashes white during the countdown

At the end of the countdown:

- Neopixel changes to green to indicate start

- The game begins

- User interacts — Targeting & Action

As the countdown begins:

- A single target (man silhouette) moves to a random position along the x and z axis, outside the user’s direct awareness

- After the countdown, the target pops upright for a limited duration (based on the current level)

The user must:

- quickly locate the active target

- use the joystick to aim the laser

- press the limit switch (trigger) to fire

- complete the action within ‘x’ seconds (depending up on the level)

- the user gets 7 bullets for each level except the final level

- System Responds — Feedback & Validation

The system checks:

- whether the laser is detected by any of the LDR sensors on the target

- and whether the trigger (limit switch) is activated simultaneously

A hit is only registered when both conditions occur at the same time.

If the target is hit on the head:

- Target instantly drops down

- LCD displays “HIT!   +5”

- Neopixel flashes green

- Buzzer emits three short beeps

If the target is hit on the body:

- Target instantly drops down

- LCD displays “HIT!   +1”

- Neopixel flashes yellow

- Buzzer emits two short beeps

If the target is missed:

- If either the LDR does not detect the laser or the trigger is not pressed within the time window

- LCD displays “MISS!”

- Neopixel flashes red

- Buzzer emits one short beep

- Target then drops down, confirming the end of that attempt after ‘x’ seconds

- Increasing Difficulty

After each attempt:

- The target begins moving along the X and Z axes

- The target repositions to a new random location

- After a short interval, the target pops upright again

With each successful hit:

- the time duration for which the target remains upright decreases

- reaction time reduces

- gameplay intensity increases

Progressive Difficulty Structure

Early Stage:

- A single target pops up three times in each level 1-3, and remains visible for a fixed duration

- The duration decreases with each level:

  - Level 1 → target stays up for 4 seconds

  - Level 2 → 3.5 seconds

  - Level 3 → 3 seconds

You get 7 bullets to fire in each level.

The neopixel lights and LCD display the remaining bullets for each level.

Advanced Stage : Level 4
As the game progresses, the interaction becomes more complex:

- The target begins moving simultaneously along the X and Z axes  for 15 seconds

- The user must now:

  - track the moving target continuously

  - adjust aim in real time

  - shoot accurately while the target is in motion

Unlimited bullets in this level.

For however many times , you hit the target, that is your score for level 4.

Scoring System Enhancement

To further increase challenge and skill differentiation:

- Each target consists of:

  - 1 sensor on head(LDR 5mm) → 5 points

  - 2 sensors on the body clustered together for bigger area (LDRs 12mm) → 1 point each

5. Outcome — Score & Feedback

After 4 rounds, the game concludes and the system presents the user’s performance.

- The LCD displays the total score of all levels combined (e.g., “xx points”)

- The Neopixel flashes multicolor lights to signal completion and celebration

After viewing their score, the user places the controller back in its designated position on the mock shooting range to reset the game.

Note: the upper 7 lights of the neopixel are to indicate the bullets in blue and the bottom 7 indicates the hit/miss colors.

### 4. Success Criteria

- Define measurable success:

- A hit should be registered only when the LDR sensor detects the laser and the limit switch is triggered simultaneously.

- The  belt carrying the target should move smoothly and accurately within the defined boundary.

- The targets should consistently pop upright to cue the user to shoot and return back to their original downright position after each attempts

- Stepper motors reliably home to (0,0) at the start of each game session using limit switches.

- The system accurately tracks the number of bullets and points.

- User experience (easy to use, intuitive)

The system should be easy to understand and operate without external instructions.

The user should be guided through the experience using clear visual and physical cues, such as:

- Neopixel lighting

- Target pop-up actions

- LCD messages

- Reliability

All wire connections should be secure and stable to prevent interruptions.

Sensors (LDR) should provide accurate and consistent detection.

The ESP32 should handle multiple simultaneous actions efficiently, ensuring smooth and uninterrupted gameplay.

Stepper motors and servos should perform smooth and repeatable movements, including accurate homing to (0,0) using limit switches.

### 5. System Overview

- High-level explanation of system

- The system is an interactive, skill-based laser shooting game that recreates the experience of a real-world shooting range through a combination of controlled mechanical movement, sensor-based detection, and real-time feedback.

- At the start of each session, a limit switch is used to home the stepper motor, establishing a fixed (0,0) reference position. This ensures consistent initialization and allows the system to accurately control and reposition the target to random locations during gameplay.

- The user controls a laser pointer using a joystick to aim at a dynamically positioned target and fires using a trigger mechanism. The target is mounted on a motorized platform that enables movement along both the X and Z axes, allowing it to appear at varying positions within a 2D plane. The Z-axis movement (front–back) is driven by a NEMA 17 stepper motor connected to a GT2 timing belt system, on which the entire X-axis assembly is mounted, enabling forward and backward translation. The X-axis movement (left–right) is achieved using a threaded rod and nut mechanism, which converts rotational motion into linear motion to move the target horizontally. The target itself is attached to a servo motor that enables it to pop upright and drop back down, acting as a clear cue for interaction.

- A hit is registered only when the laser is detected by the LDR sensors on the target at the same time as the trigger is activated, ensuring precise and intentional interaction. The system further enhances gameplay through a scoring mechanism, where the center sensor awards higher points compared to the outer sensors, encouraging accuracy.

- The system provides immediate feedback through visual (LCD, Neopixel) and auditory (buzzer) cues, reinforcing user actions and performance. As the game progresses, the difficulty increases by reducing the duration for which the target remains visible and introducing motion-based challenges, requiring the user to transition from basic reaction to precision and real-time tracking. After completing all rounds, the system displays the user’s performance and resets for replay.

- What are the main parts?

1. Controller Unit

- 1 Joystick — used to control the aiming direction via servos (X and Y axes)

- 1 Limit Switch — used as a trigger to shoot

- 1 ir sensor —detects the presence of the controller

2. Laser Pointer Mechanism (Fixed on Board)

- 1 Laser Module — emits the laser beam

- 2 Servo Motors — control movement:

  - One for horizontal (X-axis) rotation

  - One for vertical (Y-axis) rotation

3. Conveyor Belt Z-Axis

- 1 NEMA 17 Motor — drives the z axis movement of the conveyor belt to and fro.

- GT2 Belt

- Limit switch to home it down

4. Conveyor Belt X-Axis

- 1 NEMA 17 Motor — drives the x axis movement by a linear rail shaft

- 2 smooth rods

- 1 threaded rod

- Limit switch to home it to the left

- Target Unit

- 1 Servo Motor — controls the pop-up and drop-down motion of the target

- 1 LDR Sensor (head, +5 points) — detects the laser hit based on light intensity

- 2 LDR Sensors (body, +1 point) — detects the laser hit based on light intensity

6. Feedback & Output Components

- 1 Neopixel ring 16bit — provides visual feedback (color-coded signals)

- 1 LCD Screen 16x2 — displays instructions, hit/miss status, and score

- 1 Buzzer — provides auditory feedback for hits and misses

ITERATIONS

- Initially, we planned to use four LDR sensors per target, distributed across three concentric rings to assign different scores—3 points for the bullseye, 2 for the middle ring, and 1 for the outermost ring. However, this approach isn’t reliable in practice. Since the laser beam travels across the target, it would inevitably trigger the outer rings before reaching the centre. This makes it difficult to establish a clear condition where only a single, accurate score is registered, even if the player is aiming precisely at the bullseye.

- Another iteration involved moving the targets in front of the user, requiring them to aim and shoot the laser at a dynamic target. However, we realized this approach introduced a flaw—since the targets are in motion, they could eventually pass through a stationary laser beam, registering a hit even if the user didn’t actively aim or track the target. This undermines the skill-based aspect of the game and makes the scoring unreliable.

  - To address these issues and reduce the processing load on the ESP, we simplified the system to use a single LDR placed at the center of the target. To further improve accuracy, a shot is only registered when two conditions are met simultaneously: the laser is detected by the LDR and the limit switch is pressed. This acts as a form of two-step verification, ensuring that only intentional and correctly aligned shots are counted.

### 9. Failure Reflection

One night before the showcase on 21/04/2026, our NEMA 17 motors, which were supposed to control our X and Z axes, started malfunctioning. Due to this, the entire idea of the moving target collapsed.

We tried fixing the motor’s jittering, torque issues, and clockwise–anticlockwise movement with multiple efforts, even with the kind assistance of Mr. Rupin, who was working in the woodworking lab at the time. But despite our efforts, we couldn’t make it work and hence resorted to a different game idea.

We were devastated, as all our time and effort had gone into this, and it came to an unfortunate end due to a component failure, especially one that is a rare motor and couldn’t be acquired from any other student or ordered in time.

Hence, we decided to work with whatever we had in the limited time, and make the best use of our resources to make another interactive game! This experience taught us a lot in terms of patience, reliability in electronics and last minute solutionizing to get the best output.